| Subsystem | Block | Component | Status |
| --- | --- | --- | --- |
| Antenna | 2×2 RHCP Patch Array | Custom / Candidate RHCP GPS L1 Patch (+3 dBi, λ/2 ≈ 9.5 cm spacing) | Candidate |
| RF Front-End ×4 | Pre-LNA SAW BPF | Murata SAFE-B1G57KE0F00 | Locked |
| RF Front-End ×4 | LNA | Qorvo QPL9547 | Locked |
| RF Front-End ×4 | Post-LNA SAW BPF | Taoglas DBP.1567.S.A.50 | Locked |
| RF Front-End ×4 | Directional Sense Coupler | Mini-Circuits ZFDC-20-5+ | Locked |
| Vector Modulator ×4 | I/Q Vector Modulator | Analog Devices AD8341 | Locked |
| Combiner | 4-Way Wilkinson Combiner | Custom/Mini-Circuits ZAPD-4-S+(for reference) | Locked |
| Analog Feedback | Power Detector | Analog Devices AD8318 | Locked |
| Analog Feedback | MCU | STM32H74321 | Candidate |
| Analog Feedback | DAC (8-Ch) | Analog Devices AD5676 | Locked |
| Analog Feedback | Buffer Op-Amp | TI OPA354 | Locked |
| Digital Sensing | SDR / RFIC | Ettus USRP B210 (AD9361 inside) | Locked |
| Digital Sensing | Sensing ADC | AD9361 Internal 12-bit ADC | Locked |
| Clock Sync | Coherent Clock Distributor | Ettus OctoClock-G CDA-2990 | Locked |
| FPGA / DSP | FPGA + ARM | Zynq SoC Platform | Candidate |
| FPGA DSP | AoA / Nulling Algorithms | MUSIC + MVDR + Adaptive Beamforming | Locked |